RayNet

Architecture & Methodology Reference

Stereo Gaze Estimation, Iris Landmark Detection & Pupillometry

| Dataset | GazeGene (CVPR 2025) |
| --- | --- |
| Backbone | RepNeXt-M3 / M4 |
| Primary Tasks | Iris landmarks + Optical axis |
| Application | Behavioural Analysis Toolkit |

# 1. Problem Framing & Prior Work Critique

The original RayNet experiments revealed a set of compounding problems that explain the suboptimal results. This section documents each issue, its root cause, and the chosen fix.

## 1.1 The Four Core Problems

| Problem | Root Cause | Impact on Results |
| --- | --- | --- |
| Head-gaze used instead of optical axis | Bug in Experiment 2 loss target | 30° angular error — metric is meaningless |
| Weak perspective / no depth | MediaPipe assumption inherited | 12cm pupil center error at 500cm depth |
| L2 loss on 3D coordinates | GazeGene uses L1; L2 amplifies depth outliers | Unstable convergence of 3D tasks |
| 5-task MTL with MGDA/PCGrad | Over-ambitious scope before foundation solid | Gradient conflict, slow convergence |
| Direct coordinate regression for iris | No spatial distribution modelling | ~95px iris 2D error vs 1.84px baseline |

✖ CRITICAL The single highest-impact fix is correcting the gaze target: use the per-eye optical axis (eyeball center → pupil center unit vector), not the head gaze vector. This alone should drop angular error from ~30° to ~6°.

## 1.2 GazeGene Baseline We Are Targeting

All improvements are benchmarked against GazeGene's own ResNet-18 baseline (Table 5 in the paper), reproduced below for reference.

| Metric | ResNet-18 | ResNet-50 | Our Target (RayNet) |
| --- | --- | --- | --- |
| Iris 2D (px) | 1.84 | 1.73 | < 1.3 |
| Optical axis (°) | 4.98 | 4.72 | < 4.0 |
| Eyeball 3D (cm) | 0.11 | 0.13 | < 0.09 |
| Pupil 3D (cm) | 0.15 | 0.16 | < 0.12 |
| Parameters (M) | 11.7 | 23.5 | 7.8 (M3) or 13.8 (M4) |
| Latency target (ms) | ~15 | ~25 | < 10 (edge) |

# 2. Data Pipeline — Normalization & Ground Truth Construction

## 2.1 Why Normalization Solves Depth Ambiguity

Without normalization, the network must simultaneously infer (a) where in 3D space the eye is, and (b) which direction it is looking. These two unknowns are entangled in the 2D projection. Normalization removes (a) entirely by constructing a virtual camera that always views the eye center from a canonical distance dnorm = 600mm with a canonical focal length.

The result is that in normalized image space, the eye center is always at a fixed known depth. 3D coordinate regression becomes well-conditioned, and the appearance network only needs to solve direction — exactly why GazeGene's ResNet-18 achieves 0.11cm eyeball error in normalized space versus your 14.88cm in raw space.

✔ KEY INSIGHT Normalization is applied PER FRAME using the current head pose from 6DRepNet. Users can move freely — each frame gets its own warp. This is not a one-time calibration step.

### 2.1.1 Normalization Algorithm

def normalize_sample(image, K, R_head, t_eye, d_norm=600, f_norm=960, size=224):

# Scale: move eye to canonical distance

z_actual = np.linalg.norm(t_eye)

S = np.diag([1, 1, d_norm / z_actual])

# Rotation: virtual camera faces eye directly

z_axis = t_eye / z_actual

x_axis = np.cross([0,1,0], z_axis); x_axis /= np.linalg.norm(x_axis)

y_axis = np.cross(z_axis, x_axis)

R_norm = np.stack([x_axis, y_axis, z_axis])   # (3,3)

# Canonical intrinsics

K_norm = np.array([[f_norm,0,size/2],[0,f_norm,size/2],[0,0,1]])

# Perspective warp

M = K_norm @ R_norm @ S @ np.linalg.inv(K)

img_norm = cv2.warpPerspective(image, M, (size, size))

# Gaze GT transformation into normalized space:

# g_norm = R_norm @ R_head @ g_world

return img_norm, R_norm

def denormalize_gaze(gaze_norm, R_norm):

g = R_norm.T @ gaze_norm

return g / np.linalg.norm(g)   # back to camera space

## 2.2 Ground Truth Optical Axis (Fixing the Bug)

GazeGene provides both the visual axis and the kappa angles per subject. The optical axis (what the network should predict) is derived via equation (2) from the paper:

R_opt = R_vis * R_kappa^{-1}     (paper eq. 2)

In practice, the simplest and most reliable way to get the ground truth optical axis is directly geometric — it does not require constructing rotation matrices:

def ground_truth_optical_axis(subject, sample):

# Direct geometric derivation — most reliable

eyeball_center = subject['eyecenter_L']       # (3,) WCS

pupil_center   = sample['pupil_center_3d_L']  # (3,) WCS

optical_axis   = pupil_center - eyeball_center

optical_axis  /= np.linalg.norm(optical_axis)

# Sanity check vs rotation-matrix derivation

R_vis  = rotation_from_vector(sample['visual_axis_L'])

R_kap  = build_R_kappa(subject['L_kappa'])    # see Section 3

R_opt  = R_vis @ np.linalg.inv(R_kap)

axis_check = R_opt[:, 2]

assert np.allclose(optical_axis, axis_check, atol=0.01)

return optical_axis

⚠ WARNING Always use the OPTICAL axis as the training target — not the head gaze and not the visual axis. The visual axis requires knowledge of kappa, which is subject-specific and unavailable at inference without calibration.

## 2.3 Dataset Loader Changes

class GazeGeneDataset(Dataset):

def __getitem__(self, idx):

image, sample, subject, cam = self._load_raw(idx)

# 1. Locate eye center in camera coords

t_eye = cam['R_mat'] @ subject['eyecenter_L'] + cam['T_vec']

# 2. Per-frame normalization

img_norm, R_norm = normalize_sample(

image, cam['intrinsic_matrix'], cam['R_mat'], t_eye

)

# 3. Correct optical axis target

optical_axis_world = ground_truth_optical_axis(subject, sample)

optical_axis_norm  = R_norm @ cam['R_mat'] @ optical_axis_world

# 4. Iris landmarks in normalized image space

iris_2d_norm = warp_points(

sample['iris_2d_L'], R_norm, cam['intrinsic_matrix']

)

return {

'image':         img_norm,               # (3, 224, 224)

'optical_axis':  optical_axis_norm,      # (3,) unit vector

'iris_2d':       iris_2d_norm,           # (10, 2) pixels

'eyeball_3d':    subject['eyecenter_L'], # (3,) cm — WCS

'pupil_3d':      sample['pupil_center_3d_L'],

'R_norm':        R_norm,                 # for denormalization

'R_kappa':       build_R_kappa(subject['L_kappa']),

}

# 3. Kappa Angle — Anomaly, Proper Handling & Effect

## 3.1 The Anomaly

GazeGene stores L_kappa and R_kappa as np.array(3,) — three Euler angles — but the paper explicitly states kappa has no roll component. The third element is always zero and should be zeroed out before constructing any rotation matrix. Failing to do so introduces an unpredictable torsional component that distorts gaze direction.

## 3.2 Why Kappa Has No Roll — Anatomy

Kappa is the angular offset between the optical axis and the visual axis. The fovea (which defines the visual axis) is displaced from the optical axis in the nasal-superior direction within the retinal plane. This is a purely 2D offset in the Eyeball Coordinate System (ECS) — there is no torsional (roll) component by definition.

| ECS Axis | Kappa Component |
| --- | --- |
| X (nasal) | ~+4.0° horizontal kappa (population mean) |
| Y (superior) | ~+1.0° vertical kappa (population mean) |
| Z (optical axis) | 0° — NO roll, always zero |

## 3.3 Implementation

def build_R_kappa(kappa_angles):

"""

kappa_angles: np.array(3,) from GazeGene.

ALWAYS zero out roll (index 2) before use.

"""

yaw   = kappa_angles[0]   # horizontal, ~4 deg

pitch = kappa_angles[1]   # vertical,   ~1 deg

roll  = 0.0               # HARD-ZERO — ignore kappa_angles[2]

Ry = np.array([[ np.cos(yaw), 0, np.sin(yaw)],

[ 0,           1, 0           ],

[-np.sin(yaw), 0, np.cos(yaw)]])

Rx = np.array([[1, 0,             0            ],

[0, np.cos(pitch), -np.sin(pitch)],

[0, np.sin(pitch),  np.cos(pitch)]])

return Ry @ Rx   # (3,3), no roll

def optical_to_visual(optical_axis, R_kappa):

"""

At INFERENCE: convert predicted optical axis to visual axis.

R_kappa comes from calibration or population mean.

"""

visual = R_kappa @ optical_axis

return visual / np.linalg.norm(visual)

## 3.4 Effect on Gaze Accuracy

| Kappa Handling | Systematic Gaze Error | Requires Subject Data? | Recommended For |
| --- | --- | --- | --- |
| Ignore kappa entirely | ~4° per eye | No | Never |
| Spurious roll in kappa | Unpredictable | Yes (wrong) | Never |
| Population mean kappa | ~1-2° residual | No | Zero-calibration deployment |
| GT kappa (training) | Minimal | Yes (dataset) | Training only |
| Calibrated per-person kappa | Near-zero systematic | Yes (calibration) | Production toolkit |

ℹ NOTE During training, use GT kappa from the dataset to construct the correct optical axis target. At inference, use either population mean kappa or calibrated kappa depending on whether the user goes through the calibration flow.

# 4. Backbone Selection — RepNeXt-M3 vs M4

## 4.1 Architecture Comparison

| Property | RepNeXt-M3 | RepNeXt-M4 | Notes |
| --- | --- | --- | --- |
| Parameters | 7.8M | 13.8M | M4 is ~77% larger |
| Channel widths (C1-C4) | 64-128-256-512 | 64-128-256-512 | Identical channel dims |
| Depth (blocks per stage) | Shallower | Deeper | M4 adds blocks, not channels |
| Head pose MAE (AFLW2000) | ~3.91° | ~3.91° | Same perf — M3 suffices for pose |
| Suitable for iris landmarks | Yes (with PANet) | Better fine-detail (deeper) | FPN compensates for M3 |
| Edge latency (iPhone 12) | Not tested for pose | ~8.7ms (RepVGG equiv) | M3 estimated < 6ms |
| Recommended | Baseline / edge | Production / accuracy | Context-dependent |

## 4.2 The Decisive Factor: Task Complexity

Your two tasks have very different feature requirements:

- Optical axis regression: needs global context (whole eye region, head pose context). Coarse features from C4/P5 suffice. M3 is adequate.

- Iris landmark heatmaps: needs fine-grained spatial resolution at the pixel level. The landmark head attaches to P2 (stride=4, 112×112). Here, M4's additional depth helps — but PANet already provides this by fusing features from all scales.

- Pupil size (geometric post-processing): not a learned task, so backbone capacity is irrelevant here.

## 4.3 Recommendation

✔ KEY INSIGHT Start with RepNeXt-M3. Your PANet neck compensates for the shallower backbone on iris landmarks. If iris 2D error plateaus above 1.5px after proper training, upgrade to M4. The modular backbone abstraction you built in RayNet makes this a one-line change.

| Scenario | Recommendation |
| --- | --- |
| Initial baseline experiment | M3 — matches GazeGene ResNet-18 capacity |
| Edge / mobile deployment | M3 — sub-6ms latency |
| Accuracy-first (research) | M4 — deeper features for fine iris structure |
| Final production toolkit | M3 trained → distilled; upgrade if iris error > 1.5px |

# 5. Revised RayNet Architecture

## 5.1 Scope Reduction — From 5 Tasks to 2

The original Experiment 1 attempted five simultaneous tasks: gaze vector, gaze point, gaze depth, head pose, and pupil center. This caused gradient interference and made the loss landscape poorly conditioned early in training. The revised architecture focuses on exactly two tasks that directly serve the behavioural analysis toolkit.

| Task | Justification |
| --- | --- |
| Task A: Iris + pupil landmark heatmaps | Directly gives pupil diameter (geometric), iris ellipse (depth proxy), iris contour (screen projection anchor) |
| Task B: Optical axis regression | Directly gives gaze direction; combined with depth from iris gives full 3D gaze ray |
| REMOVED: Head pose task | 6DRepNet already handles this as a separate pretrained model. No need to retrain jointly. |
| REMOVED: Gaze depth task | Replaced by geometric depth from iris ellipse size — more accurate, no learning needed |
| REMOVED: Gaze point task | Derived analytically from Task B + depth once both are accurate |

## 5.2 Full Architecture Diagram

Input: Normalized eye crop (3 × 224 × 224)

│

▼

RepNeXt-M3 Backbone (4 stages)

├── C1: stride=4  → (64, 112, 112)   fine spatial

├── C2: stride=8  → (128,  56,  56)

├── C3: stride=16 → (256,  28,  28)

└── C4: stride=32 → (512,  14,  14)   global semantic

│

▼

PANet Neck (lateral 1×1 + top-down + bottom-up fusion)

├── P2: (256, 112, 112)  ← used by landmark head

├── P3: (256,  56,  56)

├── P4: (256,  28,  28)

└── P5: (256,  14,  14)  ← used by gaze head

│

├──────────────────────────────────────────────┐

▼                                              ▼

Coordinate Attention(P2)              Coordinate Attention(P5)

│                                              │

▼                                              ▼

Iris/Pupil Landmark Head               Optical Axis Head

Conv(256→128) + BN + ReLU              GAP → FC(256→128)

Heatmap branch: Conv(128→14)            FC(128→2)  [pitch, yaw]

Offset branch:  Conv(128→28)            → unit 3D vector

│                                              │

▼                                              ▼

Soft-argmax → 14 landmark coords (px)   Gaze direction (normalized space)

[10 iris pts + 4 pupil pts]                      │

│                                              ▼

▼                              denormalize: g_cam = R_norm.T @ g_norm

Iris ellipse fit (Fitzgibbon)                     │

│                                              ▼

├── Pupil diameter (mm)        Apply kappa → Visual axis

└── Depth Z (mm)                              │

│                                      ▼

└──────────────► 3D Gaze Ray → Screen coordinate (u,v)

## 5.3 Coordinate Attention Module

Coordinate Attention (CA) is the preferred attention block for mobile devices. Unlike CBAM, it encodes spatial direction information (row and column) with depthwise convolutions — preserving geometric context critical for landmark localisation.

class CoordinateAttention(nn.Module):

def __init__(self, ch, reduction=32):

super().__init__()

mid = max(8, ch // reduction)

self.pool_h = nn.AdaptiveAvgPool2d((None, 1))  # H × 1

self.pool_w = nn.AdaptiveAvgPool2d((1, None))  # 1 × W

self.conv1  = nn.Conv2d(ch, mid, 1)

self.bn1    = nn.BatchNorm2d(mid)

self.act    = nn.Hardswish()

self.conv_h = nn.Conv2d(mid, ch, 1)

self.conv_w = nn.Conv2d(mid, ch, 1)

def forward(self, x):

B, C, H, W = x.shape

h = self.pool_h(x)                         # (B,C,H,1)

w = self.pool_w(x).permute(0,1,3,2)        # (B,C,W,1)

y = torch.cat([h, w], dim=2)               # (B,C,H+W,1)

y = self.act(self.bn1(self.conv1(y)))

h, w = torch.split(y, [H, W], dim=2)

w = w.permute(0,1,3,2)

return x * torch.sigmoid(self.conv_h(h)) * torch.sigmoid(self.conv_w(w))

## 5.4 Iris / Pupil Landmark Head with Soft-Argmax

class IrisPupilLandmarkHead(nn.Module):

"""

14 landmarks: 10 iris contour + 4 pupil boundary.

Heatmap branch trains spatial distribution.

Offset branch refines to subpixel accuracy.

"""

def __init__(self, in_ch=256, n=14):

super().__init__()

self.heatmap = nn.Sequential(

nn.Conv2d(in_ch, 128, 3, padding=1), nn.BatchNorm2d(128), nn.ReLU(),

nn.Conv2d(128, n, 1))

self.offset  = nn.Sequential(

nn.Conv2d(in_ch, 128, 3, padding=1), nn.BatchNorm2d(128), nn.ReLU(),

nn.Conv2d(128, n*2, 1))

def forward(self, feat):

hm  = self.heatmap(feat)           # (B, 14, H, W)

off = self.offset(feat)             # (B, 28, H, W)

coords = self._soft_argmax(hm, off) # (B, 14, 2)

return coords, hm

def _soft_argmax(self, hm, off):

B, N, H, W = hm.shape

flat   = hm.view(B,N,-1)

weight = F.softmax(flat * 100, dim=-1).view(B,N,H,W)

gx = torch.arange(W, dtype=torch.float32, device=hm.device)

gy = torch.arange(H, dtype=torch.float32, device=hm.device)

x  = (weight * gx[None,None,None,:]).sum(dim=[2,3])

y  = (weight * gy[None,None,:,None]).sum(dim=[2,3])

# Add offset for subpixel refinement

off2 = off.view(B,N,2,H*W)

idx  = (y.long()*W + x.long()).clamp(0, H*W-1)

dx   = off2[:,:,0,:].gather(2, idx.unsqueeze(2)).squeeze(2)

dy   = off2[:,:,1,:].gather(2, idx.unsqueeze(2)).squeeze(2)

return torch.stack([x+dx, y+dy], dim=-1)

## 5.5 Loss Functions

def landmark_loss(pred_hm, pred_coords, gt_coords, sigma=2.0):

"""Heatmap MSE + coordinate L1."""

B, N, H, W = pred_hm.shape

gt_hm = gaussian_heatmaps(gt_coords, H, W, sigma)  # (B,N,H,W)

hm_loss    = F.mse_loss(torch.sigmoid(pred_hm), gt_hm)

coord_loss = F.l1_loss(pred_coords, gt_coords)

return hm_loss + coord_loss

def angular_loss(pred_gaze, gt_gaze):

"""L1 on angle — more robust than cosine for large errors."""

cos_sim = F.cosine_similarity(pred_gaze, gt_gaze, dim=-1).clamp(-1,1)

return torch.acos(cos_sim).mean()   # radians; convert to degrees for logging

def total_loss(pred_hm, pred_coords, pred_gaze,

gt_coords, gt_gaze,

lam_lm=1.0, lam_gaze=0.5):

lm  = landmark_loss(pred_hm, pred_coords, gt_coords)

gz  = angular_loss(pred_gaze, gt_gaze)

return lam_lm * lm + lam_gaze * gz

# 6. Pupil Radius Reliability for Cognitive Load Measurement

## 6.1 Why Metric Pupil Size Matters

Pupil dilation is a direct, involuntary marker of cognitive load, emotional arousal, and sympathetic nervous system activation (dilator pupillae / fight-or-flight) versus parasympathetic state (sphincter pupillae / rest-and-digest). For the CVLT and oblique effect experiments, pupil size must be expressed in metric units (mm), not pixels, because:

- Pixel size varies with subject distance from camera — two identical dilation events look different in pixels

- Cognitive load literature reports dilation in mm (baseline ~3mm, peak cognitive load ~5-6mm)

- Blink and illumination artefacts must be identified in absolute units to apply consistent thresholds

## 6.2 Geometric Pipeline — No Learned Depth Needed

Once accurate iris and pupil landmarks are available, pupil diameter in mm follows from basic projective geometry. The iris acts as a known-size ruler in the scene:

# Population mean iris radius = 5.9mm (Goldsmith et al.)

# GazeGene provides subject-specific iris_radius — use when available

def metric_pupil_diameter(iris_pts_2d, pupil_pts_2d, K,

iris_radius_mm=5.9):

"""

Returns: (Z_mm, pupil_diam_mm)

Z_mm         — depth of iris plane from camera

pupil_diam_mm — metric pupil diameter

"""

# Step 1: Fit ellipses (Fitzgibbon algebraic fit)

iris_coeff  = fit_ellipse_algebraic(iris_pts_2d)   # guaranteed ellipse

pupil_coeff = fit_ellipse_algebraic(pupil_pts_2d)

_, a_iris, b_iris   = ellipse_to_axes(iris_coeff)

_, a_pupil, b_pupil = ellipse_to_axes(pupil_coeff)

# Step 2: Depth from known iris size

# Z = f * R_iris / apparent_radius

f = K[0,0]  # focal length in pixels

apparent_iris_r = np.sqrt(a_iris * b_iris)   # geometric mean of semi-axes

Z_mm = (f * iris_radius_mm) / apparent_iris_r

# Step 3: Metric pupil radius

apparent_pupil_r = np.sqrt(a_pupil * b_pupil)

pupil_diam_mm    = (2 * apparent_pupil_r * Z_mm) / f

return Z_mm, pupil_diam_mm

## 6.3 Error Budget Analysis

The reliability of pupil measurement depends on the error chain below. With your target landmark accuracy of <1.3px iris error:

| Error Source | Magnitude | Impact on pupil (mm) | Mitigation |
| --- | --- | --- | --- |
| Iris landmark error (current) | ~95px | ~2.1mm (unusable) | Fix with heatmap head |
| Iris landmark error (target) | < 1.3px | < 0.03mm | Heatmap + soft-argmax |
| Iris radius population variation | ±0.3mm (std) | ±0.03mm depth error | Use GazeGene per-subject iris_radius |
| Depth estimation (geometric) | < 0.5% at 500mm | < 0.015mm | Accurate iris ellipse fit |
| Subject motion blur | Varies | 0.05–0.2mm | Temporal filtering (see below) |
| Partial blink occlusion | Unpredictable | 0.1–0.5mm | Occlusion flag + interpolation |
| Combined (target conditions) | — | < 0.1mm total | Comparable to desktop eye trackers |

✖ CRITICAL At the target landmark accuracy of <1.3px, geometric pupil measurement reaches ~0.03mm precision — well within the clinical threshold for pupillometry (~0.1mm). The current 95px landmark error makes pupillometry completely unreliable. Fixing the landmark head is the critical path.

## 6.4 Temporal Filtering for Cognitive Load Signal

Raw frame-by-frame pupil diameter is noisy. For cognitive load measurement, the signal of interest is the slow modulation (1–3 second timescale) driven by the locus coeruleus-norepinephrine system. Apply the following pipeline:

class PupilSizeTracker:

def __init__(self, fps=30, blink_threshold_mm=1.5):

self.fps = fps

self.raw_buffer   = collections.deque(maxlen=fps*5)  # 5s window

self.clean_buffer = collections.deque(maxlen=fps*5)

def update(self, pupil_mm, ear):  # ear = eye aspect ratio (blink detect)

"""EAR < 0.2 = blink — reject sample."""

if ear < 0.2:

return None  # blink frame

self.raw_buffer.append(pupil_mm)

# Outlier rejection: Hampel identifier

arr = np.array(self.raw_buffer)

med = np.median(arr)

mad = np.median(np.abs(arr - med))

if abs(pupil_mm - med) > 3 * 1.4826 * mad:

return None  # outlier

self.clean_buffer.append(pupil_mm)

# Savitzky-Golay smoothing (preserves peak shape)

if len(self.clean_buffer) >= 15:

smoothed = scipy.signal.savgol_filter(

list(self.clean_buffer), window_length=15, polyorder=3

)

return smoothed[-1]

return pupil_mm

def cognitive_load_index(self, baseline_mm):

"""

Pupil dilation index (PDI) relative to individual baseline.

PDI = (current - baseline) / baseline * 100  (%)

Typical cognitive load response: PDI 5-20%

"""

if len(self.clean_buffer) < 10:

return None

current = np.mean(list(self.clean_buffer)[-10:])

return (current - baseline_mm) / baseline_mm * 100

## 6.5 Expected Reliability Comparison

| System | Pupil accuracy | Suitable for cognitive load? |
| --- | --- | --- |
| Desktop eye tracker (Tobii) | ~0.05mm | Yes — gold standard |
| RayNet at target (< 1.3px iris) | < 0.1mm | Yes — comparable for task |
| RayNet current (95px iris error) | ~2mm+ | No — noise > signal |
| MediaPipe (weak perspective) | Pixel-only, no metric | No |
| OpenFace 2 (your baseline) | ~0.5mm (noisy, jittery) | Marginal — needs filtering |

# 7. Calibration Strategy

## 7.1 What Calibration Corrects

- Subject-specific kappa angle variation (±2–3° std across population)

- Camera placement relative to screen (physical geometry)

- Individual corneal refraction variation

- Any systematic model bias toward a subject's anatomy

## 7.2 Lightweight 5-Point Protocol (~5 seconds)

The user is shown a sequence of 5 dots — four corners and a centre. Each dot is displayed for 800ms with a 200ms gap. Total duration: ~5 seconds. This is framed as 'starting the session', not as a calibration task.

CALIBRATION_POINTS = [   # normalised screen coordinates

(0.1, 0.1),  # top-left

(0.9, 0.1),  # top-right

(0.5, 0.5),  # centre

(0.1, 0.9),  # bottom-left

(0.9, 0.9),  # bottom-right

]

def calibrate(raw_gaze_per_point, screen_pts):

"""Fit affine correction: screen_actual ≈ A @ raw_gaze + b"""

predicted, actual = [], []

for raw_preds, target in zip(raw_gaze_per_point, screen_pts):

# Robust median — reject blinks and saccades

med = np.median(raw_preds, axis=0)

mad = np.median(np.abs(raw_preds - med), axis=0)

clean = raw_preds[np.all(np.abs(raw_preds-med) < 3*mad, axis=1)]

if len(clean) >= 3:

predicted.append(np.mean(clean, axis=0))

actual.append(target)

X = np.hstack([predicted, np.ones((len(predicted),1))])

params, _, _, _ = np.linalg.lstsq(X, actual, rcond=None)

A, b = params[:2].T, params[2]

return A, b   # apply as: corrected = raw @ A.T + b

## 7.3 Zero-Calibration (Passive Mode)

For passive behavioural tasks where interrupting the user is unacceptable (e.g. CVLT), use the population mean kappa and a screen-geometry prior. This introduces ~1–2° systematic error but requires no user interaction. Opportunistic refinement can be performed silently when natural fixations are detected (e.g. user clicks at a known screen location).

ℹ NOTE For cognitive load measurement, the relative pupil dilation signal (PDI%) is more important than absolute gaze accuracy. Zero-calibration is acceptable for pupillometry tasks even if screen-mapping precision is reduced.

# 8. Training Methodology — Step-by-Step Plan to Beat GazeGene Baseline

## 8.1 Stage 0 — Verify Data Pipeline (before any model work)

Incorrect data loading has been responsible for all suboptimal results so far. Before training a single epoch, verify every component of the pipeline:

- Optical axis sanity check: For each sample, verify that the geometric optical axis (pupil - eyeball normalized) matches the rotation-matrix-derived axis to within 0.01 radians.

- Kappa roll check: Assert kappa_angles[2] ≈ 0 for all subjects. Log any outliers.

- Normalization invertibility: Normalize a frame, denormalize the GT gaze, reproject to 2D, verify it falls within 2px of the original GT 2D gaze point.

- Landmark coordinate check: Plot normalized iris landmarks over normalized eye crop for 20 random samples. They should form a consistent ellipse around the iris boundary.

## 8.2 Stage 1 — Reproduce GazeGene Baseline Exactly

Before optimizing, reproduce the GazeGene paper's exact ResNet-18 results. This validates the pipeline and gives a reliable comparison point.

| Setting | Value |
| --- | --- |
| Backbone | ResNet-18 (exact, no modifications) |
| Input | Normalized 224×224 face crop |
| Loss | L1 on eyeball 3D, pupil 3D, iris 2D, + angular on optical axis |
| Optimizer | Adam, β=(0.5, 0.95), lr=1e-4 |
| Epochs | 10 |
| Batch size | 512 |
| Train/test split | Subjects 1–46 / 47–56 (as per paper) |
| Normalization | Yes — Zhang et al. 2018 |
| Target (iris 2D) | < 2.0px (within 10% of paper's 1.84px) |
| Target (optical axis) | < 5.5° (within 10% of paper's 4.98°) |

✖ CRITICAL If you cannot reproduce within 10% of the paper's numbers, the data pipeline has a bug. Do not proceed to Stage 2 until Stage 1 passes.

## 8.3 Stage 2 — Swap Backbone to RepNeXt-M3 + PANet

Keep all training settings identical to Stage 1. Replace only the backbone and add the PANet neck. This isolates the contribution of the architectural improvement.

| Change | Expected Effect |
| --- | --- |
| ResNet-18 → RepNeXt-M3 | Similar capacity, inference-time reparameterisation, ~30% faster |
| Global pooling → PANet P2+P5 | Fine-grained features for iris; global features for gaze |
| Add Coordinate Attention on P2/P5 | Geometry-aware attention; ~0.1–0.3° gaze improvement |
| Keep direct coordinate regression for now | Isolate backbone effect before changing loss |

## 8.4 Stage 3 — Heatmap Landmarks (Key Improvement)

Replace direct iris coordinate regression with heatmap + soft-argmax head. This is the change most likely to beat the GazeGene baseline on iris 2D accuracy.

| Hyperparameter | Value |
| --- | --- |
| Gaussian sigma for GT heatmaps | 2.0px (start); anneal to 1.0px by epoch 15 |
| Softmax temperature in soft-argmax | 100 (start high for stable gradients) |
| Loss weights | λ_heatmap=1.0, λ_coord=1.0, λ_gaze=0.5 |
| Landmark head input feature | PANet P2 (stride=4, 112×112) |
| Gaze head input feature | PANet P5 (stride=32, 14×14) |

## 8.5 Stage 4 — Progressive Training Schedule

Phase 1  (epochs  1-5):  warm up landmark head only

λ_landmark = 1.0,  λ_gaze = 0.0,  lr = 1e-3

Rationale: establish iris/pupil spatial priors before gaze

Phase 2  (epochs  6-15): introduce gaze gently

λ_landmark = 1.0,  λ_gaze = 0.3,  lr = 5e-4

Rationale: 2D projection anchors 3D gaze direction

Phase 3  (epochs 16-30): balance and fine-tune

λ_landmark = 0.5,  λ_gaze = 0.5,  lr = 1e-4

Scheduler: CosineAnnealingLR(T_max=15)

## 8.6 Stage 5 — Multi-View Consistency Loss

Add cross-view reprojection consistency using all 9 camera angles. This is a self-supervised signal unique to GazeGene that the paper's own baseline does not use — a genuine differentiator for RayNet.

λ_consistency = 0.2   # start small; increase if stable

# For each training batch, sample 2 random cameras from the 9.

# Project camera-i landmarks into camera-j using GT depth.

# Penalise discrepancy with camera-j predictions.

consistency_loss = F.l1_loss(

reproject(lm_cam_i, cam_i, cam_j, depth_i),

lm_cam_j.detach()   # stop-gradient on target side

)

## 8.7 Projected Results

| Stage | Iris 2D (px) | Optical Axis (°) | Notes |
| --- | --- | --- | --- |
| GazeGene ResNet-18 (target) | 1.84 | 4.98 | Paper baseline |
| Stage 1 — reproduce | < 2.0 | < 5.5 | Pipeline validation |
| Stage 2 — RepNeXt+PANet | ~1.6 | ~4.5 | Architecture swap |
| Stage 3 — heatmap head | ~1.2 | ~4.2 | Key improvement |
| Stage 4 — progressive train | ~1.1 | ~3.8 | Curriculum effect |
| Stage 5 — consistency loss | ~1.0 | ~3.5 | Multi-view bonus |

# 9. Stereo RayNet & Knowledge Distillation — When and How

## 9.1 Sequencing Rationale

The stereo teacher + monocular student architecture is the right long-term design. However, it must be built on a working monocular foundation. Attempting distillation from a teacher with 30° gaze error propagates wrong representations to the student.

| Milestone required before stereo | Status |
| --- | --- |
| Optical axis bug fixed (head gaze → per-eye) | TODO |
| Normalization implemented | TODO |
| Stage 1 reproduction passed | TODO |
| Iris 2D error < 1.5px | TODO |
| Optical axis error < 5° | TODO |
| All above complete → build stereo teacher | FUTURE |

## 9.2 Stereo Teacher Architecture (When Ready)

class StereoTeacher(nn.Module):

"""Processes all 9 views; fuses via cross-view attention."""

def __init__(self, backbone, n_cams=9, d=256):

super().__init__()

self.backbone   = backbone

self.cam_embed  = nn.Linear(12, d)  # encode 3x4 extrinsics

self.cross_attn = nn.MultiheadAttention(d, num_heads=8, batch_first=True)

self.lm_head    = IrisPupilLandmarkHead(d)

self.gaze_head  = GazeHead(d)

def forward(self, imgs, cam_params):

# imgs: (B, 9, 3, 224, 224)

feats = []

for i in range(9):

f = self.backbone(imgs[:,i]).mean(dim=[2,3])  # (B, d)

cam_enc = self.cam_embed(cam_params[i].flatten(-2))  # (B, d)

feats.append(f + cam_enc)

feats = torch.stack(feats, dim=1)   # (B, 9, d)

fused, _ = self.cross_attn(feats, feats, feats)

agg = fused.mean(dim=1)              # (B, d)

return self.lm_head(agg), self.gaze_head(agg)

## 9.3 Distillation Loss

def distillation_loss(student_lm, student_gaze,

teacher_lm, teacher_gaze,

gt_lm,     gt_gaze,

alpha=0.7):

"""alpha controls teacher vs GT balance."""

hard = (F.l1_loss(student_lm, gt_lm) +

angular_loss(student_gaze, gt_gaze))

soft = (F.l1_loss(student_lm, teacher_lm.detach()) +

angular_loss(student_gaze, teacher_gaze.detach()))

return alpha * soft + (1 - alpha) * hard

# 10. Decision Summary

| Decision | Choice | Rationale | When |
| --- | --- | --- | --- |
| Backbone | RepNeXt-M3 | 7.8M params; PANet compensates for shallower depth; upgrade to M4 if iris > 1.5px | Now |
| Neck | PANet (YOLOv8-style) | Multi-scale fusion; P2 for landmarks, P5 for gaze | Now |
| Attention | Coordinate Attention | Geometry-aware; lightweight; better than SE/ECA for spatial tasks | Now |
| Tasks | 2 only: landmarks + optical axis | Reduce gradient conflict; cover all downstream needs geometrically | Now |
| Landmark loss | Heatmap MSE + L1 coord | Subpixel accuracy; occlusion-robust; beats direct regression | Stage 3 |
| Gaze loss | L1 angular on optical axis | Correct target (not head gaze); L1 robust to outliers | Stage 1 |
| Coordinate space | Normalized (Zhang 2018) | Removes depth ambiguity; per-frame with head pose | Stage 0 |
| Depth estimation | Geometric from iris ellipse | No learned depth; ~0.5% error; uses known iris radius | Post-training |
| Pupil size | Geometric from iris+pupil ratio | Metric mm output; no additional learning | Post-training |
| Kappa | Build R_kappa with zero roll | Anatomically correct; prevents torsional distortion | Stage 0 |
| Calibration | 5-point affine (~5 seconds) | Corrects per-person kappa and camera placement | Deployment |
| Stereo teacher | Defer until mono < 5° | Cannot distil from broken teacher | Future |

Document generated from design discussions covering the full RayNet architecture revision. All code snippets are reference implementations — verify against the active repository branch.